Purpose

The purpose of the lab is to demonstrate the signal analysis capabilities of Matlab.  The oscilloscope will be used as an A/D converter to capture several signals we have examined in previous labs.  These signals will then be examined and analyzed in Matlab.  Effects of A/D quantization noise will be seen.

Reference Sources

Proakis chapter 7.4.1, equations 7.4.1 to 7.4.3 as attached

Prelab

- YOU WILL NEED thumb drive

- What is the quantization noise power Pq (eq 7.4.2) for an 8-bit A/D with an input range of  +1V to -1V? (n=8 and xmax=1V)

- Does this noise power depend on the sample frequency?  Explain

- The power spectral density, PSD, of the noise is Sq=PqFs2 Watts/Hz.  This is the quantization noise floor of the A/D.  What is the noise floor PSD when Fs is 10Ks/sec?

- What would the Signal to quantization noise ratio SQNR be if the input signal is a 2Vp-p sine wave, xmax=1V, and n=8 bits?   (SQNR=Psignal/Pq)

Capture the following signals on a floppy disk as csv files for analysis in Matlab.  You may do the data analysis at your leisure before the next lab.

1.	Set the oscilloscope for a sample rate of 100 Ks/sec, 10,000 points, and 1mV/div.  Disconnect the probe from the scope.  This is the minimum resolution of the scope.

- Record the mean, Pk-Pk, and RMS voltages using the scope measurements

- Capture the trace to a  .csv type data file

- Write a Matlab file to read the data file and:

  - Plot the data in time

  - Find the mean, Pk-Pk and RMS noise voltages.  Compare the results to what you recorded on the oscilloscope.

  - Plot the power spectral density (PSD) in Watts/Hz.  Be sure to correct for the resolution bandwidth of the FFT.  You may assume R=1

  - Estimate the voltage resolution of the A/D) by finding the minimum non-zero voltage change between points.

  - The full scale range of the A/D is x 512 (9-bit).  How does this compare to the full scale range of 8 mV as seen on the scope screen?  Explain

2.	Set the oscilloscope for a sample rate of 100 Ks/sec, 10,000 points, and 200mV/div.  Disconnect the probe from the scope.

- Capture the trace to a data file

- Repeat part 1 using this data.

3.	Set the oscilloscope for a sample rate of 100 Ks/sec, 10,000 points, and 200mV/div. Now connect a signal generator and apply a 1 KHz sine wave of 1 Vp-p.

- Capture the trace to a data file.

- Repeat part 1 using this data.

- Estimate the full scale range of the scope, +/1 Xmax, under these settings.  How does this compare to the displayed +/- 800 mV signal on the scope display.

- Estimate the dynamic range of the scope in dB, SNQR=20*log(Xmax/Vnoise). Vnoise is the no input rms noise voltage from 2.

- Compare the level of the noise you expect to see for the sine wave with what you see in the FFT plot.  Explain. (Hint: Resolution Bandwidth)

4.	Set the oscilloscope for a sample rate of 100 Ks/sec, 10,000 points, and 200mV/div. Now connect a signal generator and apply a 1 KHz sine wave of 2 Vp-p.

- Capture the trace to a data file.

- Repeat part 1 using this data.

- Is the sine wave distorted in the Matlab plots?  Explain

5.	Change the input waveform to a 100 KHz triangle waveform 2.5 Vp-p

- Capture the trace to a data file.

- Repeat part 1 using this data.

- Is the triangle wave distorted in the Matlab plots?  Use this to find the pk-pk range of the A/D.

Pq=

Sample data analysis from Matlab program:

Screen shots from EELE44512lab5.m Matlab file.  You should see something similar.

Zoom was used on the above graph to see the individual quantization steps. 4mV in this case.